Explosion (HTB) Walkthrough

1. Reconnaissance

An initial full TCP port scan was performed against the target system to identify exposed network services.

nmap -sS -Pn -sV -p- <target-ip>

• -sS performs a TCP SYN scan to identify open ports
• -Pn disables host discovery and assumes the host is alive
• -sV enables service and version detection
• -p- scans all 65,535 TCP ports

The scan revealed that TCP port 3389 was open.

2. Service Enumeration

After identifying an open port, a targeted scan was conducted to enumerate the service running on TCP port 3389.

nmap -sS -sV -v -Pn -sC -p 3389 <target-ip>

• -sC runs Nmap default scripts for service enumeration
• -v enables verbose output

The scan identified the service as ms-wbt-server, corresponding to Microsoft Terminal Services (Remote Desktop Protocol), confirming that the target was a Windows system with RDP enabled.

3. RDP Access Attempt

Since RDP was exposed, a remote desktop connection attempt was made using the FreeRDP client.

xfreerdp /v:<target-ip> /u:Administrator

• /v:<target-ip> specifies the target host
• /u:Administrator specifies the Windows user account

The connection prompted for a password. A blank password was submitted, and authentication succeeded.

This resulted in direct remote desktop access with full graphical user interface (GUI) control, confirming that the Administrator account did not enforce password-based authentication.

4. Flag Retrieval

Once logged into the system, the flag was found directly on the desktop of the Administrator account.

The file was opened using Notepad, and the flag value was retrieved successfully.

5. Exploitation Outcome

The misconfigured RDP service allowed unauthenticated remote access to the system.

No authentication, exploitation, or privilege escalation was required to gain full administrative access and retrieve the flag.